THE NATIONAL UNIVERSITY OF LESOTHO

Department of Mathematics and Computer Science · Faculty of Science and Technology

CS3520 Computer Organization and Architecture · AY 2026/2027

# LIMO Group Project · Milestone M1 Submission

LIMO Instruction Set Architecture

| Submission field | Details |
| --- | --- |
| Team / lab group | Team name: Binary Brains Lab group: A,B,C |
| Release / commit | Release tag: m1 Commit SHA: fbdbd1b50dfcca4dbed989fdbe541ae180e02fa1 |
| Repository / live site | GitHub repository: htts://github.com/MAKHAOLA-N/LIMO_BINARY_BRAINS Live site: ______________________________________________________ |
| Team members | Makhaola Nkhahle Student no. 202403142 GitHub MAKHAOLA-N Eric Letsoela Student no. 202403632 GitHub ericletsoela Ramosamo Lekhela Student no. 202404795 GitHub lekhela Hlabathe Moeko Student no. 202408117 GitHub moekohlabathe18 Kananelo Ketlele Student no. 202401765 GitHub Kananelok Bonolo Makhotla Student no. 202404536 GitHub bonolomakhotla-a11y Ntoetse Selomo Student no. 202321292 GitHub ntoetseselemo-crypto |

# 1. Team Charter and Set-up

| Item | Team agreement / evidence |
| --- | --- |
| Project board | GitHub Projects URL: https://github.com/users/MAKHAOLA-N/projects/1/views/1 |
| Meeting rhythm | We meet everyday at 10 pm. |
| Decision rule | The team make decisions by majority vote. |
| Definition of done | A task is complete when it meets requirements, is tested, reviewed, merged into main, and documented. |
| Branch and PR rules | one task, one branch, one pull request, and at least one review before merging |
| Absence / non-contribution | The team contact them to understand the reason and remind them of their responsibilities. |

# 2. ISA Specification — Overview

| Specification | Rationale |
| --- | --- |
| 32-bit load–store; fixed 32-bit instructions; PC advances by 4 bytes. | Fixed width simplifies fetch and decode; instructions are word-aligned. |
| 16 × 32-bit registers; 4-bit indices. | Saves one bit per register operand versus RV32I; increases register pressure. |
| Register, immediate, base + byte offset, PC-relative branch addressing. | Supports arithmetic/logic, word memory access and conditional branches. |
| Byte-addressed, little-endian memory; word accesses; aligned instructions. | Simple memory model. Misaligned data addresses are rounded down in this subset. |
| Formats R, I, S, B; no U or J. | No upper-immediate or jump instruction is included in this 15-instruction subset. |
| No multiply/divide, floating point, CSR or architectural exceptions. | Limits implementation complexity for the milestone. |

## 2.2 Register File

Sixteen 32-bit registers use 4-bit indexes. r0 is hard-wired to zero; PC is separate architectural state and is not encoded in register fields.

| Number | Sesotho name / alias | Type | Purpose and convention |
| --- | --- | --- | --- |
| 0 | r0 / lefela | SPR | Constant zero; writes ignored. |
| 1–13 | r1–r13 / n1–n13 | GPR | General-purpose registers. |
| 14 | r14 / mokokotlo (sp) | SPR | Stack pointer. |
| 15 | r15 / aterese ea khutlo (ra) | SPR | Return address. |
| PC | khaontara ea lenaneo | SPR | Byte address of current instruction; dedicated, not encoded. |

## 2.3 Instruction Set and Sesotho Mnemonics

Operand syntax: R = rd, rs1, rs2; I arithmetic/logic = rd, rs1, immediate; load = rd, offset(rs1); S = rs2, offset(rs1); B = rs1, rs2, label.

| Class | Mnemonic | Meaning | Operation | Format / RV32I |
| --- | --- | --- | --- | --- |
| Arithmetic | eketsa | increase | rd = rs1 + rs2 (wraps modulo 2³²) | R / add |
| Arithmetic | fokotsa | reduce | rd = rs1 − rs2 | R / sub |
| Logic | kopanya | join together | rd = rs1 AND rs2 | R / and |
| Logic | kapa | or | rd = rs1 OR rs2 | R / or |
| Logic | fapana | differ | rd = rs1 XOR rs2 | R / xor |
| Logic | suthisa | shift | rd = rs1 << rs2[4:0] | R / sll |
| Logic | khutlisetsa | bring back | rd = rs1 >> rs2[4:0], logical | R / srl |
| Compare | nyane | small/less | rd = 1 if signed rs1 < rs2, else 0 | R / slt |
| Arithmetic | eketsaI | add immediate | rd = rs1 + sign-extend(imm14) | I / addi |
| Logic | kopanyaI | AND immediate | rd = rs1 AND sign-extend(imm14) | I / andi |
| Logic | kapaI | OR immediate | rd = rs1 OR sign-extend(imm14) | I / ori |
| Memory | jarisa | load | rd = Mem[rs1 + sign-extend(imm14)] (one word) | I / lw |
| Memory | ngola | write | Mem[rs1 + sign-extend(imm14)] = rs2 (one word) | S / sw |
| Branch | lekana | equal | if rs1 == rs2: PC = PC+4+(sign-extend(disp14)<<2) | B / beq |
| Branch | fapane | differ | if rs1 != rs2: PC = PC+4+(sign-extend(disp14)<<2) | B / bne |

Lexical rules. ASCII apostrophe (U+0027) is allowed and preserved in mnemonics, labels and register aliases to preserve Sesotho spelling (e.g., ts' and ch'). Curly apostrophes and other non-ASCII punctuation are rejected with a line-numbered error. Current mnemonics contain no apostrophes; the rule supports labels and future additions.

## 2.4 Instruction Encoding Specification

Bit 31 is most significant. Register indexes are unsigned 4-bit values. Reserved fields are written as zero by the assembler and ignored by hardware.

| Format | Bit fields from bit 31 to bit 0 | Total |
| --- | --- | --- |
| R | opcode[31:26] · rd[25:22] · rs1[21:18] · rs2[17:14] · funct[13:6] · 0[5:0] | 6+4+4+4+8+6 = 32 |
| I | opcode[31:26] · rd[25:22] · rs1[21:18] · imm[17:4] · 0[3:0] | 6+4+4+14+4 = 32 |
| S | opcode[31:26] · rs1[25:22] · rs2[21:18] · imm[17:4] · 0[3:0] | 6+4+4+14+4 = 32 |
| B | opcode[31:26] · rs1[25:22] · rs2[21:18] · disp[17:4] · 0[3:0] | 6+4+4+14+4 = 32 |

Field decisions and departures from RV32I

| Change | LIMO decision | Reason / trade-off |
| --- | --- | --- |
| Register index | 4 bits, 16 registers (RV32I: 5 bits, 32) | Saves one bit per register operand and narrows forwarding comparators; only 13 registers remain for general values. |
| Opcode | 6 bits at [31:26] (RV32I: 7 bits at [6:0]) | Top-aligned class decode; each I, S and B instruction has its own opcode. Not binary-compatible with RV32I. |
| Immediate | 14-bit signed field at [17:4] in I/S/B | Common position permits one sign extender; range −8192 to +8191. |
| R function | 8-bit funct at [13:6] | Allows 256 register-register operations; 8 are currently used. |
| Reserved bits | [3:0] zero in I/S/B; [5:0] zero in R | Kept for extensions; hardware ignores them. |
| Branch target | PC + 4 + (sign-extend(disp14) << 2) | Word displacement removes always-zero low bits and gives ±32 KiB reach. |
| Formats / memory | R, I, S, B only; word loads/stores only | No U/J needed; simple memory unit. Misaligned data address rounds down because exceptions are absent. |

Immediate ranges

| Use | Range | Interpretation |
| --- | --- | --- |
| I arithmetic/logic | −8192 to +8191 | Sign-extended value |
| I load / S store | −8192 to +8191 | Byte offset from rs1 |
| B branch | −8192 to +8191 words | −32768 to +32764 bytes from PC+4 |
| Shift amount | rs2[4:0] | Low five bits determine shift amount. |

Opcode assignments: R=000000; eketsaI=000001; kopanyaI=000010; kapaI=000011; jarisa=000100; ngola=000101; lekana=000110; fapane=000111. R funct values: eketsa=00000001, fokotsa=00000010, kopanya=00000011, kapa=00000100, fapana=00000101, suthisa=00000110, khutlisetsa=00000111, nyane=00001000. Other opcodes and unused funct values are reserved.2.5 Encoding Worked Examples

Examples transcribed from the specification. Binary is shown as a 32-bit word, with fields in order from bit 31 to bit 0. Verify these encodings against the assembler before final release.

| Format / instruction | Field values | Binary (bit 31 … 0) | Hex |
| --- | --- | --- | --- |
| R eketsa r3,r1,r2 | op 000000; rd 0011; rs1 0001; rs2 0010; funct 00000001; zero 000000 | 00000000110001001000000001000000 | 0x00C48040 |
| R fokotsa r5,r4,r3 | op 000000; rd 0101; rs1 0100; rs2 0011; funct 00000010; zero 000000 | 00000001010100001100000010000000 | 0x0150C080 |
| I eketsaI r4,r0,12 | op 000001; rd 0100; rs1 0000; imm 00000000001100; zero 0000 | 00000101000000000000000011000000 | 0x050000C0 |
| I jarisa r3,0(r1) | op 000100; rd 0011; rs1 0001; imm 00000000000000; zero 0000 | 00010000110001000000000000000000 | 0x10C40000 |
| S ngola r2,8(r1) | op 000101; rs1 0001; rs2 0010; imm 00000000001000; zero 0000 | 00010100010010000000000010000000 | 0x14480080 |
| B lekana r1,r2,fela | op 000110; rs1 0001; rs2 0010; disp 00000000000011; zero 0000 | 00011000010010000000000000110000 | 0x18480030 |

Branch example: if label fela is 16 bytes after the branch instruction, displacement = (16 − 4) / 4 = 3 words from PC+4. Branch reach is −32768 to +32764 bytes from PC+4.

Important encoding check

The field layouts above are the agreed LIMO layouts. Because source drafts may contain hand-encoding or alignment mistakes, the team should independently re-derive each binary/hex example and make the assembler reproduce the checked words exactly before tagging m1.3. Design-Decision Log

Each rationale is grounded in the proposed LIMO programs and pipeline design. Confirm against actual implementation and measured results before submission.

- Register-file size

Sixteen 32-bit registers use 4-bit indexes instead of RV32I’s 5-bit indexes. An R-type instruction with three register operands saves three encoding bits, used to provide wider function fields. Forwarding comparators also narrow from 5 to 4 bits. The file holds 512 bits; r0 is constant zero and r14/r15 are reserved for stack pointer/return address, leaving 13 general-purpose registers. This increases register pressure; larger routines may require register reuse or spills to memory.

- Architectural or microarchitectural?

The PC and r0–r15 are architectural because their values and behavior are visible to programs. r0 is hard-wired zero, r14 is the stack pointer, r15 the return address, and r1–r13 general-purpose. IR and pipeline registers (IF/ID, ID/EX, EX/MEM, MEM/WB) are microarchitectural: they hold intermediate pipeline data and may change with implementation. The ISA must not specify them so different pipeline designs can execute the same program with the same architectural behavior.

- Branch resolution

LIMO resolves conditional branches in EX after comparing source registers. If taken, younger instructions in IF and ID are on the wrong path and are flushed: maximum penalty two instructions. Resolving in ID could reduce flushing but requires earlier comparison and extra forwarding/bypass paths. EX resolution is chosen for simpler datapath and hazard handling, accepting the two-instruction taken-branch penalty. (d) Load-use stall

A load-use hazard costs one stall even with full forwarding: the loaded value appears only at the end of MEM, while the dependent instruction needs it at the start of EX. The hazard unit holds PC and IF/ID and inserts a bubble into ID/EX. In the next cycle the value is forwarded from MEM/WB to EX. The array-sum loop contains this pair, so each iteration pays one stall.

| Instruction | C1 | C2 | C3 | C4 | C5 | C6 | C7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| jarisa r3, 0(r1) | IF | ID | EX | MEM | WB |  |  |
| eketsa r4,r3,r2 |  | IF | ID | ID (stall) | EX | MEM | WB |
| bubble |  |  |  | EX | MEM | WB |  |

3. Design-Decision Log (continued)

- A flags register?

LIMO has no flags register. Branches compare two source registers directly, avoiding implicit shared state and allowing independent comparisons. A flags register would add writer-reader dependencies when a branch reads flags produced by an earlier instruction, creating extra RAW hazards and potentially requiring hazard detection, forwarding or stalls. Direct comparison keeps the ISA and hazard handling simpler.

- What breaks first as programs grow?

Register pressure is expected first: only 13 registers remain for general values, and an array-sum loop already needs a pointer, limit, accumulator and temporary. Immediate range comes second: signed 14-bit values cover −8192 to +8191, and larger constants need an instruction sequence because there is no upper-immediate instruction. Branch reach comes last at roughly ±32 KiB (8192 instructions each way).

# 4. Sample LIMO Programs

Program 1 — Arithmetic. Demonstrates immediate and register arithmetic.

eketsaI r1, r0, 7 eketsaI r2, r0, 5 eketsa r3, r1, r2 # r3 = 12 fokotsa r4, r1, r2 # r4 = 2

Expected final state: r1=7, r2=5, r3=12, r4=2.

Program 2 — Logic and shift. Demonstrates bitwise operations and shifting.

eketsaI r1, r0, 10 eketsaI r2, r0, 3

kopanya r3, r1, r2 # AND = 2 kapa r4, r1, r2 # OR = 11 fapana r5, r1, r2 # XOR = 9 suthisa r6, r2, r2 # 3 shifted left by 3 = 24

Expected final state: r3=2, r4=11, r5=9, r6=24.

Program 3 — Array sum. Sum three preloaded words.

Initial memory: memory[0]=4, memory[4]=−2, memory[8]=7 eketsaI r1,r0,0 # base address eketsaI r2,r0,3 # item count eketsaI r3,r0,0 # index eketsaI r4,r0,0 # running sum loop: lekana r3,r2,done

jarisa r5,0(r1) eketsa r4,r4,r5 eketsaI r1,r1,4 eketsaI r3,r3,1

lekana r0,r0,loop # unconditional branch because r0==r0 done: ngola r4,12(r0)

Expected final state: r4=9; memory[12]=9. Confirm label syntax, branch encoding and loop behavior in the simulator.

References

- Patterson, D. and Hennessy, J. Computer Organization and Design: RISC-V Edition. Morgan Kaufmann.

- Waterman, A. and Asanovi■, K. (eds). The RISC-V Instruction Set Manual, Volume I: Unprivileged ISA (RV32I). RISC-V International.

Sesotho mnemonics: verify with the team’s Sesotho sources/speakers: __________________________

AI-Use Declaration

| Tool | Used for | How checked / changed |
| --- | --- | --- |
| ChatGPT | Explaining RV32I formats and template sections | Compared with RISC-V manual and lecture notes; specification text written by team. |
| Claude AI | Reviewing register file, immediates, branch reach | Recomputed ranges and encodings by hand; fixed issues identified. |
| Textbook.( Computer Organization and Design) | to explain concepts, provide examples. | The team reviews the work, tests it, and makes corrections |

Sprint goal: Finalize and document the LIMO ISA so every instruction has a verified encoding and every design choice has a rationale.

| Memeber | Role | Delivered(issue and PR members) | PRs reviewed |
| --- | --- | --- | --- |
| Makhaola Nkhahle | ISA lead: overview, register file | ISA lead: overview, register file #1 | None |
| Eric Letsoela | Design-decision log: Section 3 (a) to (f), load-use diagram | Design-decision log #2 | None |
| Bonolo Makhotla | Instruction set: table, Sesotho mnemonics | Sesotho mnemonics #3 | None |
| Kananelo Ketlele | Formats and encoding: formats, immediate ranges, worked examples | Formats and encoding #4 | None |
| Hlabathe Moeko | Sample programs: Section 4, hand-traced expected final states | Sample programs #7 | None |
| Ntoetse Selomo | Repo and PR coordinator: board, branch rules, tag m1, docs/M1.pdf | Repo and PR coordinator #5 | None |
| Ramosamo Lekhela | cross-checks encodings against the table, references, AI-use declaration, final checklist | references, AI-use declaration #6 | None |

Not done and why:

Next sprint: Milestone 2

Blockers: Translating English to Sesotho