CS3520 · LIMO Simulator · Milestone M2 · Team Binary Brains

Member Contribution: ISA Architect

Bonolo Makhotla

| Role | ISA architect |
| --- | --- |
| Student no. | 202404536 |
| GitHub | bonolomakhotla-a11y |
| Team | Binary Brains |
| Responsibility | The LIMO instruction set as specified in M1, kept frozen in M2 |

# 1. Summary of Role

I am responsible for the LIMO instruction set architecture (ISA). I specified it in Milestone M1 and kept it frozen in M2, so the assembler and simulator implement exactly the ISA that was defined and defended. In total the ISA has 16 registers, 15 instructions and 4 fixed-width 32-bit formats. The six worked encodings in the M1 specification are reproduced exactly by the assembler.

# 2. Deliverables

- Register file: 16 registers of 32 bits, with fixed roles for r0, r14 and r15.

- Instruction set: fifteen instructions across the R, I, S and B formats.

- Instruction formats: four fixed-width 32-bit layouts with defined field sizes.

- Limits and conventions: immediate range, branch displacement and memory addressing rules.

- Verification: six worked encodings that the assembler reproduces exactly.

- Freeze record: a statement that the ISA is unchanged from M1, with the Defence 1 feedback table.

# 3. Register File

The ISA has 16 registers, each 32 bits wide. Three have fixed roles:

| Register | Name | Role |
| --- | --- | --- |
| r0 | lefela | Always zero |
| r14 | mokokotlo | Stack pointer (sp) |
| r15 | aterese ea khutlo | Return address (ra) |

# 4. Instruction Set

There are fifteen instructions, grouped by format.

| Format | Count | Instructions |
| --- | --- | --- |
| R-type | 8 | eketsa, fokotsa, kopanya, kapa, fapana, suthisa, khutlisetsa, nyane |
| I-type | 4 | eketsaI, kopanyaI, kapaI, jarisa |
| S-type | 1 | ngola |
| B-type | 2 | lekana, fapane |
| Total | 15 |  |

# 5. Instruction Formats

All instructions are 32 bits wide. Field widths are in bits.

R format

| Field | opcode | rd | rs1 | rs2 | funct | unused |
| --- | --- | --- | --- | --- | --- | --- |
| Bits | 6 | 4 | 4 | 4 | 8 | 6 |

I, S and B formats

| Field | opcode | reg 1 | reg 2 | immediate / displacement | unused |
| --- | --- | --- | --- | --- | --- |
| Bits | 6 | 4 | 4 | 14 | 4 |

# 6. Limits and Conventions

| Item | Definition |
| --- | --- |
| Immediates | Signed 14-bit, range −8192 to 8191 |
| Branch displacement | Counted in words, relative to PC+4 |
| Memory | Byte-addressed, little-endian |

# 7. Verification and Freeze

- Worked encodings: The assembler reproduces the six worked encodings in the M1 specification exactly.

- ISA status: The ISA is unchanged from M1.

- Defence 1 feedback: A table records the Defence 1 (D1) feedback and the changes made in response.

# 8. Where to Find the Work

| File / section | Content |
| --- | --- |
| docs/M1-ISA-Specification.docx | The M1 specification: registers, formats, instructions and worked encodings. |
| docs/M2-design.md, §1 | ISA-frozen section: format table and D1 feedback table. |
| src/assembler.js (tables) | FUNCT, OPCODE, REGISTERS, immediate limits and the LAYOUT of each format. |